# Edge Hardware Comparison

| Feature | Arduino (Microcontroller) | Raspberry Pi (Single-board computer) |
| --- | --- | --- |
| CPU Type | ATmega328P (8-bit AVR RISC Single Core, 16Mhz clock) | Broadcom BCM2711 (Quad-core Cortex-A72 64-bit ARM,1.5 Ghz clock) |
| RAM Size | 2 KB SRAM 32 KB Flash | 2GB, 4GB, or 8GB LPDDR4(separate chip, not on CPU) |
| Power Source | 7-12V (Barrel Jack) or 5V (USB) | 5V DC via USB-C (min 3A) |
| Typical Use Case | Simple sensor control, basic motor driving, and real-time signal monitoring. | Mini desktop,Media centers, web servers, and complex Edge AI inference (e.g., Computer Vision). |

## Comparison Summary

- Execution Environment: The Arduino runs code directly on the hardware ("bare-metal"), whereas the Raspberry Pi runs a full operating system (typically Linux), allowing for multitasking.

- Performance: The Raspberry Pi offers significantly higher clock speeds and multiple cores, making it capable of processing heavy datasets or running neural network models.

- Resource Constraints: Arduinos are extremely resource-constrained but highly power-efficient for "always-on" simple tasks, whereas Raspberry Pis require more power but provide a desktop-like computing experience.

- Connectivity: While both have GPIO pins, the Raspberry Pi includes built-in high-speed peripherals like USB 3.0, Gigabit Ethernet, and HDMI outputs.

- Real-time Capabilities: Microcontrollers like the Arduino are better suited for tasks requiring strict real-time response times due to their simpler architecture.