# Selected Model & Optimization Technique

| Category | Selection |
| --- | --- |
| Edge Model | MobileNetV2 |
| Optimization Technique | Post-Training Quantization (float32 → int8) |

## Model Analysis

- Original Model Size: MobileNetV2 typically has approximately 3.4 million parameters, which equates to roughly 14 MB when using standard 32-bit floating-point (float32) precision.

- Optimization Changes: The quantization process modifies the model's weight and activation representations from 32-bit floats to 8-bit integers (int8). It involves mapping the continuous range of floating-point values to a discrete range of 256 integer values.

- Expected Effects:

  - Size: A significant reduction in model size, typically by 4x (shrinking the 14 MB model to approximately 3.5 MB).

  - Speed: Faster inference speeds, especially on hardware with integer-optimized processing units or SIMD instructions, as integer math is computationally cheaper than floating-point math.

  - Accuracy: A minor tradeoff in accuracy (typically <1-2%) due to the loss of precision from discretization.

# Why this optimization is appropriate

- Hardware Compatibility: Many low-power edge devices and microcontrollers lack dedicated Floating Point Units (FPUs); quantization allows them to execute models efficiently using integer arithmetic.

- Memory Footprint: Reducing the model size from 14 MB to 3.5 MB is critical for fitting the entire network within the limited Flash and SRAM storage available on edge hardware.

- Power Efficiency: Integer operations require fewer clock cycles and significantly less energy per operation compared to floating-point calculations, which is vital for battery-powered IoT devices.

- Reduced Latency: By streamlining the math and reducing the amount of data moved between memory and the CPU, quantization ensures the "real-time" responsiveness required for applications like gesture or object detection.

- Bandwidth Efficiency: Smaller model weights mean less data needs to be fetched from memory during each inference cycle, further reducing the computational bottleneck on resource-constrained chips.